Summary of Recommended Biosafety Levels for Infectious Agents

| BSL | Agents | Practices | Primary Barriers and Safety Equipment | Facilities (Secondary Barriers) |
| --- | --- | --- | --- | --- |
| 1 | Not known to consistently cause diseases in healthy adults | Standard microbiological practices | No primary barriers required. PPE: laboratory coats and gloves; eye, face protection, as needed | Laboratory bench and sink required |
| 2 | Agents associated with human disease Routes of transmission include percutaneous injury, ingestion, mucous membrane exposure | BSL-1 practice plus: - Limited access - Biohazard warning signs - “Sharps” precautions - Biosafety manual defining any needed waste decontamination or medical surveillance policies | Primary barriers: - BSCs or other physical containment devices used for all manipulations of agents that cause splashes or aerosols of infectious materials PPE: Laboratory coats, gloves, face and eye protection, as needed | BSL-1 plus: - Autoclave available |
| 3 | Indigenous or exotic agents that may cause serious or potentially lethal disease through the inhalation route of exposure | BSL-2 practice plus: - Controlled access -Decontamination of all waste -Decontamination of laboratory clothing before laundering | Primary barriers: - BSCs or other physical containment devices used for all open manipulations of agents PPE: Protective laboratory clothing, gloves, face, eye and respiratory protection, as needed | BSL-2 plus: - Physical separation from access corridors - Self-closing, double-door access - Exhausted air not recirculated - Negative airflow into laboratory - Entry through airlock or anteroom - Hand washing sink near laboratory exit |
| 4 | Dangerous/exotic agents which post high individual risk of aerosol-transmitted laboratory infections that are frequently fatal, for which there are no vaccines or treatments Agents with a close or identical antigenic relationship to an agent requiring BSL-4 until data are available to redesignate the level Related agents with unknown risk of transmission | BSL-3 practices plus: - Clothing change before entering - Shower on exit - All material decontaminated on exit from facility | Primary barriers: - All procedures conducted in Class III BSCs or Class I or II BSCs in combination with full-body, air-supplied, positive pressure suit | BSL-3 plus: - Separate building or isolated zone - Dedicated supply and exhaust, vacuum, and decontamination systems - Other requirements outlined in the text in BMBL 5th Ed. |

From 	Section IV—Laboratory Biosafety Level Criteria

Biosafety in Microbiological and Biomedical Laboratories (BMBL) 5th Edition

Available at: http://www.cdc.gov/biosafety/publications/bmbl5/